GENBN401– BASICS OF NETWORKING

DATE: Monday, 16 June 2025

PERIOD: 8.30 AM-11.30 AM

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

END OF TERM III EXAMINATIONS SCHOOL YEAR: 2024/2025

SECTOR: ICT and MULTIMEDIA RQF LEVEL: L4

…………/100

…………/70

TRADE:	SOFTWARE DEVELOPMENT MARKS:		CAMIS:

DURATION: 3 HOURS

INSTRUCTIONS TO CANDIDATES:

This Exam paper is composed of Three Sections (A, B, C). Follow the instructions given below, and answer the indicated questions for a

total of 100 marks

Section A: Seventeen (17) questions, all Compulsory

55 marks

Section B: Among the five (5) questions, attempt any three (3)	30 marks

Section C: Among the two (2) questions, attempt any one (1)

15 marks

Allowed materials:

-	Ruler ,Blue or Black pen

Level 4 GENBN401– Basics Of Networking  –	Page 1 of 20 –Marking guide

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

SECTION A: Attempt all questions	(55 marks)

| 01. | Read carefully and choose correct statements that describe the definition of “computer network is: - A collection of hardware components and computers interconnected by communication channels that aren’t allow sharing of resources and information. - A network Architecture in which each computer on the network is either a client or a server network. - A Powerful computer that provides services to other computers on the network. - A group of computers connected with each other to communicate and share information and resources like hardware, data, and software. | (2marks) |
| --- | --- | --- |
| 02. | Choose the correct answer from the following listed alternatives. - A network is…………… - One or more computer connected for sharing resources - Two or more computer connected for sharing resources - A series of telephone lines - What is the key benefit of using a network? - Sharing information and resources - It is cheap - Security | (4marks) |

Level 4 GENBN401– Basics Of Networking  –	Page 2 of 20 –Marking guide

| 03. | Choose the best standard for IP Address from the flowing standard? - Internet protocol address - Ethernet protocol address - Intranet protocol address - Internet property address | (2marks) |
| --- | --- | --- |
| 04. | Select the Type of IP Addresses from the following options: A. Prevalent B. Public C. Internet D. protocol | (2marks) |
| 05. | Differentiate between Internet Protocol (IP) and Domain Name in a network and give one example for each? | (4marks) |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 3 of 20 –Marking guide

| 06. | Which of the following is a type of maintenance that will involve in repairing or replacing a damaged network cable? - Preventive maintenance - Corrective maintenance C. Software maintenance D. Firmware maintenance | (3marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 4 of 20 –Marking guide

| 07. | Which of the following is the main purpose of upgrading network monitoring software? - To fix bugs and enhance security - To clean physical devices - To change device passwords - To configure new network adapter | (3marks) |
| --- | --- | --- |
| 08. | From the listed Networking hardware and network Infrastructure corrective in network devices maintenance, separate them in their respective column. | (4marks) |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

| Networking hardware and network Infrastructure | Networking hardware | network Infrastructure |
| --- | --- | --- |
| - Firewall and Gateway - Coaxial cable - Switch - Fiber-optic cable - Access Point - Ethernet cables - Router - Antenna Wi-Fi |  |  |

| 09. | After understanding IP Address Classification, answer the questions bellow: - Which of the following is the primary function of an IP address? - To store data packets - To identify a device and enable communication over a network - To encrypt data over the network - To assign internet speed to a device - Which of the following IP address types is assigned by an ISP for temporary use and can change over time? - Static IP address - Private IP address - Dynamic IP address - Public IP address | (4marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 6 of 20 –Marking guide

| 10. | After Understanding IP Versions, provide the answer of the following questions: - Which of the following is a feature of IPv4? - 128-bit addressing scheme - Supports a larger number of unique IP addresses than IPv6 - Uses hexadecimal notation for IP addresses - Creates 32-bit IP addresses - Which version of IP uses a 128-bit hexadecimal addressing scheme? - IPv1 - IPv4 - IPv6 - IPv3 | (4marks) |
| --- | --- | --- |
| 11. | Which of the following should you do if the network card is detected as an "Other device" in Windows Device Manager? - Restart the computer and try again - Download the latest drivers for the network card - Replace the network card - Check the router’s configuration | (3marks) |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 7 of 20 –Marking guide

| 12. | What is the major purpose of making recommendations in a network maintenance report? - To highlight the network's weaknesses - To suggest improvements or enhancements for future maintenance - To critique the network administrator's performance - To focus on hardware replacement | (3marks) |
| --- | --- | --- |
| 13. | Which of the following is used to prevent signals from bouncing back on a coaxial cable? A. T-type connector B. Terminator connector C. BNC barrel connector D. F connector | (3marks) |
| 14. | Classify the IP addresses below based on their range and name of classes. Those IP address are: - 192.168.112.99 - 32.110.21.18 c. 168.168.168.99 d. 223.118.99.88. | (4marks) |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 8 of 20 –Marking guide

| 15. | Which of the following steps correctly leads you to configure a network adapter to use a dynamic IP address (DHCP) through the Control Panel? - Open Control Panel, > click on Network Center, > select "Change adapter settings, > click on property, > Connect internet." - Open Control Panel, > click on Network and Internet, > choose "Network and Sharing Center", > Go to the property, > shared internet. - Open Control Panel, > click on Network and Sharing Center, > click "Change adapter settings." - Open Control Panel, > click on Network and Internet, > connect to the internet, > view network status. | (3marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking  –	Page 9 of 20 –Marking guide

| 16. | By using a table, classify the following IP addresses into their appropriate network classes (class A, class B, class C, class D). Those IP addresses are: 225.1.0.2, 100.30.70.9, 230.10.23.2, 10.10.10.10, 130.1.0.100, 200.4.6.2, 172.16.0.10, 193.200.4.9 | (4marks) |
| --- | --- | --- |
| 17. | Which of the following steps are necessary to set a static IP address in Windows 10 or 11? - Open the Start Menu > Control Panel > Network and Sharing Center > Change adapter settings. - Right-click on Wi-Fi > Local Area Connection > select Properties>Contral panel>View network status>Task. - Select Internet Protocol >Version 4 (TCP/IPv4)>click Properties>View network and internet. D. Select "Use the following IP address" >enter the necessary IP address> Subnet mask>Default gateway. | (3marks) |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 10 of 20 –Marking guide

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Section B: Attempt any three (3) questions	(30 marks)

| 18. | Match these types of networks with their roles in networking (Write role number in the Answer column) Type of Roles Answer network - WAN A. It is a small-scale network that revolves ……. around one person or device. It connects just a few devices in a small localized area. - GAN B. It is designed to connect multiple …… network devices and systems within a limited geographical distance. - MAN C. It is a network type that covers the ……. network connection of an entire city or connection of a small area. - PAN D. It is designed to connect devices over …….. large distances like states or between countries. The connection is wireless in most cases and uses radio towers for communication. - LAN E. It is network that covers a global or ……. international scale, often using satellite links and undersea cables. | (10marks) |
| --- | --- | --- |

Level 4 GENBN401– Basics Of Networking –	Page 12 of 20 –Marking guide

| 19. | Given the IP address 192.168.2.143/22, Answer the following | (10marks) |
| --- | --- | --- |
|  | questions: |  |
|  | A. Identify the class of this IP address. |  |
|  | B. Determine the network ID and host ID for this IP address. |  |
|  | C. What is the subnet mask associated with this network? |  |
|  | D. How many bits are used for the network portion? |  |
|  | E. How many hosts can be accommodated within this network? |  |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 13 of 20 –Marking guide

| 20. | Network media is the physical path through which data travels from one device to another in a network. It can be a wire (like a cable), Network cables include RJ45 cables, Coaxial Cables among others. - Identifying the necessary materials for making RJ45 cable - Develop a step-by-step guide on how to make an RJ45 cable | (10marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 14 of 20 –Marking guide

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

| 21. | Consider the window below that was derived after running an IPCONFIG command in a laboratory, Evaluate the significance and implications of the information displayed and answer related sub questions - Which IP Version 4 address this computer is connected to? - In which class it belongs to? - What is its subnet mask? - Which IPv6 that this WLAN adapter is using - What is its default gateway? - How many bits which specify IPv4 and IPv6 addresses? | (10marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 16 of 20 –Marking guide

| 22. | Subnetting is the process of dividing a larger network (IP address block) into smaller, more manageable pieces called subnets. Differentiate between Fixed Length Subnet Masking (FLSM) and Variable Length Subnet Masking (VLSM). | (10marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 17 of 20 –Marking guide

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Section C: Attempt only one (1) question	(15 marks)

| 23. | You’ve been assigned to set up a network for a small business. To build an efficient, reliable, and scalable system, which five physical network topologies would you consider for connecting devices like computers, printers, and servers? For each topology: A. Explain how it works. B. Provide a simple diagram to illustrate the connection setup. | (15marks) |
| --- | --- | --- |

Level 4 GENBN401– Basics Of Networking –	Page 18 of 20 –Marking guide

| 24. | IP address assignment is the process of allocating unique Internet Protocol (IP) addresses to devices on a network so they can communicate with each other and access the internet or internal services. To allocate IP addresses effectively, you need to complete a | (15marks) |
| --- | --- | --- |

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Level 4 GENBN401– Basics Of Networking –	Page 19 of 20 –Marking guide

table below that outlines important details for the three primary IP address classes (Class A, Class B, Class C). For each class, provide:

- The First Network ID

- The Last Network ID

- The Default Subnet Mask

- Identify which class would be most appropriate for large, medium, and small network environments.

KIREHE ADVENTIST TVET SCHOOL(APAPEN)

Use this table format to complete your responses:

| CLASS | First Network ID | Last Network ID | Default Subnet mask | Typical Use Case |
| --- | --- | --- | --- | --- |
| Class A | ? | ? | ? | ? |
| Class B | ? | ? | ? | ? |
| Class C | ? | ? | ? | ? |